Bugs in usbhidio Firmware

Below are a list of bugs and fixes that have been reported for my usbhidio firmware. Eventually I hope to incorporate these into the code, but until then, here are the bugs and what to do about them. Thanks to all who contributed. If you have bug reports for any of my code examples, please send them to me at jan@lvr.com.

Jan Axelson

http://www.lvr.com

jan@lvr.com

***

Problem: Stall returned in Control Write transfers on some PCs

Solution:in the subroutine StageOne, the StatusOuts bit is set right in the beginning. I used the same approach but its not correct for the machines with OHCI host. These host send the Out data packet immediately after the Setup packet (control write transfer) and since the StatOuts bit is set the CY7C63001A SIE responds with STALL (received Out Data packet is not Out Status packet).To fix this bug just set this StatOuts bit to 1 at the end of Setup packet ISR.

***

Problem:

I have a question that in the EP0 section,

;If no, find out if it's an OUT packet.

jz check_for_out_packet

check_for_out_packet:

iord USB_EP0_RX_Status

;Is it an OUT packet?

and A, 02h

;If no, ignore it.

jz done_with_packet

done_with_packet:

;Re-enable Endpoint 0 interrupts.

mov A,[interrupt_mask]

or A, 08h

mov [interrupt_mask], A

ipret Global_Interrupt

Why there is no need to sendstall if neither SETUP packet or OUT packet is found?

Solution: Add code to return a Stall if it's not a Setup or Out packet.

***

Problem: The code sometimes does a watchdog timeout during enumeration.

Solution: The "kludge" fix is to kick the dog in the 1ms_isr, but you rightly wanted to take that out from what you say the code history. Or, add watchdog resets in the wait loops of waitcontrolread and waitfordatatotransfer.

***

Problem:

If a descriptor is a multiple of 8 bytes long, you have to transmit a zero length packet at the end, so that the host knows that you've finished.

Right now the code doesn't do that; as it stands it doesn't matter because there are no such descriptors, but its a problem when you start editing the report.

Solution: In Control_Read:

;If there is no data packet, do the handshake.

mov A, [data_count]

cmp A, 00h

;          jz control_read_status_stage        ;CHANGE THIS LINE

jz dma_load_done                                        ;TO THIS LINE

***

Problem: The main loop may fail due to the way the OUT packets are handled in the Endpoint 0 interrupt.    All OUTs are going through this code, even those which are not part of the SET RECORD command.    In fact, during enumeration there are about 14 OUTs.    They all get stuck in this code and stop the main loop from executing.    (Your code doesn’t use the main loop, but any practical application will need it.)

Solution: The following code does two things.    First, there is a separate routine to get the data sent from the host in the SET RECORD command.    This is analogous to “control_data_read” for reading descriptors but handles output instead of input.

Second, as much as possible has been moved out of the interrupts, making them as short as possible.

In the string section of the .INF file,

[Strings]

Provider="Lakeview Research"

MfgName="USB Complete"

USB\VID_0925&PID_1234.DeviceDesc="Sample USB human interface device (HID)"

the line MfgName="USB Complete" should read MfgName = “Lakeview Research”

***

Problem: In the one ms interrupt, the comments regarding loop_temp are incorrect:

;Find out if enumeration is complete.

;If enumerating is in progress, loop_temp = 0.

mov A, [loop_temp]

cmp A, 0h

;If enumeration is still in progress, jump.

jz not_main

;Enumeration has ended, so decrement the loop counter

;(so it no longer = 0).

dec [loop_temp]

Solution:

In fact, loop_temp is set to 255 at initialization.    The one ms code makes it count down to zero, then stop.    So the delay is in progress as long as loop_temp is non-zero.

Problem:

This code:

cmp A, 255

jz ResetToZero

inc A

jmp NewValueSet

ResetToZero:

mov A, 0

NewValueSet:

Solution:

is the same as

inc A