# HIDDemo2

This is the readme file for the HIDDemo2 application, which includes Visual-Basic host software and device firmware for communicating with USB HID-class devices. HIDDemo2 is an enhanced version of the usbhidio project, also available from Lakeview Research.

HIDDemo2 is especially useful for communicating with devices that send data infrequently. The application uses an ActiveX Exe server to read from the device. Because the reads are in a separate thread, the main application doesn't hang while waiting for the device to send something.

HIDDemo2 (and usbhidio) are brought to you by

Jan Axelson

Lakeview Research

2209 Winnebago St.

Madison, WI 53704

Phone: 608-241-5824

Fax: 608-241-5848

http://www.lvr.com

jan@lvr.com

## Files

HIDDemo2 includes the following files:

### The Readme File

readme.doc is this file.

### The PC’s Inf File

Usbhidio.inf is an INF file that tells Windows what drivers to use for the device. The device uses Windows’ built-in HID drivers. You can view the file in any text editor.

### The Visual-Basic Project

There are two Visual-Basic 6 projects, the application and the server. In the directory VB:

Apideclarations.bas contains declarations for the Win32 API functions used in the projects.

In the subdirectory HIDDemo2:

HIDDemo2.vbp is a Visual-Basic project that finds the device and enables the user to request the application to send reports to the device and/or receive reports from the device.

frmMain.frm is the application’s form.

In the subdirectory HIDServer:

HIDServer.vbp is a Visual-Basic ActiveX Exe server project that handles reading from the device. The reads are handled in the server because ReadFile is a blocking call. The server runs in its own thread, and can wait for the device to send data without requiring the main application to wait.

frmServer.frm is the server's form.

clsHID.cls has the routines for reading from a HID.

### The Device Code

In the directory CY7C63000:

Hid5sec.asm is assembly-code firmware for Cypress’ CY763001 USB microcontroller. The code handles enumeration on attachment to a host, and can exchange data in both directions. The firmware conforms to the USB’s human-interface device (HID) class specification. On request, the firmware will make an Input report available every 5 seconds.

Hid5sec.rom is the assembled code for loading into the development board’s memory.

Hid5sec.lst is the listing file produced by the assembler.

Hid5sec.hex is the Hex file produced by the assembler (for use with device programmers that accept Intel Hex files).

## What You Need

To use this code, you need all of the following:

- A PC running Windows 98.

- Visual Basic 6, to load HIDDemo2.vbp and HIDServer.vbp.

- Cypress’ 3650 development kit, which contains a development board, assembler, and a monitor program for loading and running the firmware, or the Starter Kit (see Tips below).

- An understanding of USB and HID communications. See http://www.lvr.com for links to more information about USB and HIDs.

### Also helpful are:

- The freeware USBCheck and HIDView utilities, available from http://www.usb.org . These will tell you if the host successfully enumerated the device, and if so, will display the descriptors retrieved. HIDView can also read data from the device.

- Oscilloscope, logic analyzer, voltmeter, protocol analyzer. The USB transmissions are encoded in a way that makes it just about impossible to interpret the data without a specialized protocol analyzer, but basic test equipment is still useful for debugging. For example, the firmware includes debug code that sets port bits when the code is in a particular routine.

- The HID documentation in the Windows 98 DDK, in the WDM section. You can download the DDK from Microsoft.

- References I used in creating the ActiveX Exe server were the Visual-Basic documentation section Creating an ActiveX Exe Component and Dan Appleman's book, Developing ActiveX Components with Visual Basic.

## How to Use the Code

1. Copy usbhidio.inf to your \Windows\Inf directory.

2. Use the Cypress monitor program to load hid5sec.rom into the development board’s memory. Press the Start button on the development board.

3. Unplug and replug the cable. Look for the device under Windows 98’s Start Menu > Control Panel > System > Device Manager > Human Interface Devices. The device should be listed as    both “HID-compliant device” and “Sample USB Human Interface Device (HID)”.

4. Run Visual Basic. Load the HIDServer.vbp project.

5. Create a reference executable: select File > Make HIDServer.exe.

6. Run with full compile: select Run > Start with Full Compile or press Ctrl + F5.

7. Run a second instance of the Visual Basic. Load the HIDDemo2.vbp project.

8. In Project > References, check HIDServer. (Use the reference described as HIDServer.vbp, not HIDServer.exe).

9. Run with full compile: select Run > Start with Full Compile or press Ctrl + F5.

10. To send and receive device reports, click the command buttons.

11. To close the project, close both the HIDDemo2 application and the HIDServer projects. The server is set for Multiuse instancing, so you can start and stop HIDDemo2 without having to stop and restart HIDServer.

12. From this point, you can edit and experiment with the Visual-Basic and Cypress code as you wish.    hid5sec.asm contains the source code for the firmware. Edit it with any text editor, save as plain text, re-assemble using Cypress’ Cyasm assembler, and use the monitor program to load the new version into the development board.

13. Send any comments, questions, bug reports, etc. to me at jan@lvr.com.

## Tips

- When you run USBCheck and HIDView, Windows detects your device as a “USB Diagnostic Device” (under Start Menu > Control Panel > System > Device Manager > Other Detected Devices. When you’re finished using the utilities, unplug and replug the USB cable to allow Windows to re-enumerate the board as a standard Human Interface Device.

- ReadFile is a blocking call. If the device has nothing to send, HIDServer will hang while it waits, but the application won't be blocked. You can click the command buttons and select values in the combo box. Clicking the Stop Receiving Data button will cause the device to send a report that enables the server to close. (There are additional comments in the source code.)

- The firmware will run on the Starter Kit. Use the EPROM programmer to load usbhidio.hex into a chip, insert the chip into the kit's socket, and attach the board to the bus. The code currently will fit in a 2k CY7C63000, or a 4k '63001.

- To prevent errors, close the Cypress monitor program before you run USBCheck or HIDView.

- If your PC doesn’t boot with the development board attached, unplug the USB cable, boot up, and reload the firmware.

- After loading new firmware on the development board, press the board's Start button, then unplug and replug the USB cable so that Windows will detect and enumerate the device with the new firmware.

- The clsHID class in HIDServer is set for Multiuse instancing (clsHID Properties > Instancing). This makes it easier for developing, because you can close and restart the application without also having to close and restart HIDServer. But the application is designed with the assumption that only one thread will be accessing HIDServer, so you should set the property to SingleUse before creating a compiled version for running outside the VB environment.

- If you try to run HIDDemo2 and get this error: Compile Error. Can't find project or library, select Project > References (it should pop up automatically), uncheck Missing: HidServer, and click OK. Then select Project > References again and click the HIDServer item described as HIDServer.vbp (not HIDServer.exe). The server must be running for this reference to appear. You then should be able to run the application.

### Tips for using Cyasm (the Cypress assembler):

- Variable names are case-sensitive. Port0_data is not the same as Port0_Data.

- The “~” signifies complement. For example, The complement of the variable “test” is “~test”. This is undocumented.

## Rights information

The computer software provided in the HIDDemo2.zip file is copyright 1999 by Jan Axelson. The software is provided as-is. You have a royalty-free right to use, modify, reproduce and distribute this software (and/or any modified version) in any way you find useful, provided that you agree that Jan Axelson and Lakeview Research have no warranty, obligation or liability for their contents!

I welcome any comments, bug reports, questions, etc. about this code. Contact me at jan@lvr.com .

Jan Axelson