HIDUART Test App

MOTO Development Group

11/8/00

# Using HIDUART.exe

Before using HIDUART.exe, you must first install an MO1002 as a standard USB Human Interface Device (HID).  To do that, simply plug the device into a USB port and accept all the default options on the New Hardware Wizard that pops up.  You may need to insert your Windows 98 CDROM if you have not previously installed a HID on your system and the Wizard prompts you to do so.  To use HIDUART.exe, an MO1002 must be plugged in to a USB port.

Click on Open to open a connection to the device.  Any ASCII data sent from the device will then appear in the lower edit control.  Any text that ends with a carriage return will also have a line feed appended, so that it will appear on separate lines in the edit control.  You can erase the text in that edit control by right-clicking on it, picking Select All, and then pressing the Delete key.

To send data to the device, enter it in the edit control to the right of the Send button, and click on that button.  A carriage return will be appended to that data.

If you unplug the MO1002 while HIDUART.exe has a connection open to it, then you will need to close that connection, by clicking on Close, and re-open it by clicking Open again, before you can send or receive data again.